Duet Module Interface Specification

for a

BSS Audio Blu50 (RPM)

Audio Mixer

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	4

Port Mapping	5

Channels	7

Levels	7

Command Control	8

Command Feedback	10

Device Notes	10

Programming Notes	10

Protocol Notes	11

Adding Functions to Modules	11

Commands to the device	11

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 01-19-16 | CGA | Initial release |

## Introduction

PLEASE NOTE THAT THIS DUET PROJECT IS MEANT FOR RAPID PROJECT

MAKER (RPM) USE.  IF ATTEMPTING TO USE THIS PROJECT IN A

NON-RPM INSTALLATION YOU MUST USE THE APPROPRIATE AUDIO

ARCHITECT CONFIGURATION FILE (BLU_50_RPM_Configuration_v1.0.0.audioarchitect) OR IT WILL NOT WORK PROPERLY!

This is a reference manual to describe the interface provided between an AMX NetLinx system and a BSS Audio Blu50 . The BSS Audio Blu50 supports an RS-232 serial and IP protocol. The interface was tested using version 86.02.2 of the firmware. The required communication settings are a baud rate of 115200, 8 data bits, 1 stop bit, no parity, and handshaking off. The cable for this device is FG#10-752. The wiring diagram for this cable is as follows:

AMX NXI	AMX NI	Third Party (DB-9 Male)

1 GND		5 GND		5 GND

2 RXD		2 RXD		2 TXD

3 TXD		3 TXD		3 RXD

This module was written using Master firmware version v3.50.439, NetLinx Studio version v3.2 build 3.2.0.418, Café Duet application platform and runtime version 1.15.230, and Café Duet application plug-in version 1.8.85.

In the case of IP control, the Comm. module must be informed of the IP address of the device it is to connect to. This must be done manually via a telnet session, or can be automated from the UI module. The IP address must be set using the ‘PROPERTY-‘ command. The ‘REINIT’ command is used to notify the Comm. module that new properties are now available and to start using them. When these two commands are used in the appropriate order, the Comm. module will attempt to connect to the specified IP address periodically until a connection is made or a new IP address is submitted using the ‘PROPERTY-‘ and ‘REINIT’ commands. Please see the Programming Notes section for additional information.

## Overview

The COMM module translates between the standard interface described below and Audio Mixer device  serial protocol. It parses the buffer for responses from the Audio Mixer device, sends strings to control the Audio Mixer device, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

{enter device make and model here}

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the AMX BSS Audio Blu50 module, the programmer must perform the following steps:

- Define the device ID for the Audio Mixer device that will be controlled.

- Define the virtual device ID that the BSS Audio Blu50 COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file BSS BLU50.TP5 and module and componentized UI modules have been created for testing.

- The Duet BSS Audio Blu50 module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the Audio Mixer device to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

dvBSSBLU50 	= 0:3:0 // ETHERNET CONTROL

//dvBSSBLU50 	= 5001:1:0 // This device should be used as the physical device by the COMM module

vdvBSSBLU50 	= 41001:1:0  // The COMM module should use this as its duet device

vdvBSSBLU502 	= 41001:2:0  // Room Volume

vdvBSSBLU503 	= 41001:3:0  // Mic Volume

vdvBSSBLU504 	= 41001:4:0  // VTC Volume

DEFINE_VARIABLE

DEV vdvDev[] = {vdvBSSBLU50}

DEV vdvDev5[] = {vdvBSSBLU50,vdvBSSBLU502,vdvBSSBLU503,vdvBSSBLU504}

// ----------------------------------------------------------

// CURRENT DEVICE NUMBER ON TP NAVIGATION BAR

INTEGER nBSSBLU50 = 1

// ----------------------------------------------------------

// DEFINE THE PAGES THAT YOUR COMPONENTS ARE USING IN THE

// SUB NAVIGATION BAR HERE

INTEGER nVideoConferencerPages[] = { 1 }

INTEGER nDialerPages[] = { 2 }

INTEGER nVolumePages[] = { 9 }

INTEGER nMenuPages[] = { 10,11,12 }

INTEGER nModulePages[] = { 13 }

DEFINE_START	// Place define_module calls to the very end of the define_start section.

// Touch panel modules

DEFINE_MODULE 'BSS BLU50 AudioConferencerComponent' audioconferencer(vdvDev, dvBSSBLU50Tp, nBSSBLU50, nVideoConferencerPages)

DEFINE_MODULE 'BSS BLU50 DialerComponent' dialer(vdvDev5, dvBSSBLU50Tp, nBSSBLU50, nDialerPages)

DEFINE_MODULE 'BSS BLU50 MenuComponent' menu(vdvDev, dvBSSBLU50Tp, nBSSBLU50, nMenuPages)

DEFINE_MODULE 'BSS BLU50 ModuleComponent' module(vdvDev, dvBSSBLU50Tp, nBSSBLU50, nModulePages)

DEFINE_MODULE 'BSS BLU50 VolumeComponent' volume(vdvDev5, dvBSSBLU50Tp, nBSSBLU50, nVolumePages)

// Comm module

DEFINE_MODULE 'BSS_BLU50_Comm_dr2_0_0' comm(vdvBSSBLU50, dvBSSBLU50)

Upon initialization the AMX Comm module will communicate with the Audio Mixer device and information will be exchanged.

## Port Mapping

This module uses multiple virtual devices in order distinguish events for one zone from another.

{Note: None may be substituted in the Channels, Levels, Control or Feedback columns as needed.}

{ Insert as many virtual devices as needed }

| Virtual Device | Channels | Levels | Control | Feedback |
| --- | --- | --- | --- | --- |
| 41001:1:0 – System Control | 251, 252 | none | All | All |
| 41001:2:0 – Room Volume | All (except 251, 252) | All | none | none |
| 41001:3:0 – Mic Volume | All (except 251, 252) | All | none | none |
| 41001:4:0 – VTC Volume | All (except 251, 252) | All | none | none |

Table 1 - Port Mapping

## Channels

The UI module controls the {device} via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

| Channel | Description |
| --- | --- |
| 24 | ON: Ramp Volume Up – used for feedback also OFF: Stop Ramping |
| 25 | ON: Ramp Volume Down – use for feedback also OFF: Stop Ramping |
| 26 | PULSE: Cycle Volume Mute |
| 199 | ON: Set Volume Mute On – used for feedback also OFF: Set Volume Mute Off |
| 251 | ON: Device is Online – used for feedback only OFF: Device is not Online |
| 252 | ON: Data is Initialized – use for feedback only OFF: Data is not Initialized |

Table 2 - Virtual Device Channel Events

## Levels

The UI module controls the {device} via level events (NetLinx command send_level) sent to the COMM module. The levels supported by the COMM module are listed below. These levels are associated with the virtual device(s) and are independent of the levels associated with the touch panel device.

| Level | Description |
| --- | --- |
| 1 | Volume Level (range 0…255) |

Table 3 - Virtual Device Level Events

## Command Control

The UI module controls the Audio Mixer device via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & info messages on 4 = set all messages on DEBUG-1 |
| PASSBACK-<state> | Enable or disable response reporting from the device. When enabled device responses will be sent as strings to the virtual device. Note: By default, this is set to off at startup. <state> : 0 = Off (default) 1 = On PASSBACK-0 |
| PASSTHRU-<string> | Allows user the capability of sending commands directly to whatever unit is attached with minimal processing by the Duet module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section below. Note: The appropriate STX, character replacement, Check Sum, ETX are automatically added by the module. <string> : string to send to unit "'PASSTHRU-',$88,$6b,$5c,$03,$00,$01,$1a,$00,$61,$00,$00,$00,$00" (Room Volume mute off) |
| ?PROPERTY-<key>> | Get the value of a property <key>. If the value is not initialized, an empty string is returned. <key> : IP_Address <key> : DEVICE_ID <key> : Poll_Time ?PROPERTY-IP_Address |
| PROPERTY-<key>,<value> | Set the value of property <key> to <value>. This must be followed by the REINIT command to take effect. These values are not initialized by default. <key> : IP_Address <value> : string representing an IP address PROPERTY-IP_Address,10.0.0.5 <key> : DEVICE_ID <value> : HiQNet Node Address ((hexadecimal representation) of the target device PROPERTY-DEVICE_ID,0x6B5C <key> : Poll_Time <value> : string representing the ping time to see if device is present (in milliseconds). Default = 30000 PROPERTY-=Poll_Time,30000 |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. REINIT |

Table 4 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for Audio Mixer device changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command | Description |
| --- | --- |
| PROPERTY-<key>,<value> | Feedback on the value of property <key>. Note: An empty string is returned if the property has no value. <key> : IP_Address <value> : string representing an IP address <key> : DEVICE_ID <value> : HiQNet Node Address ((hexadecimal representation) of the target device <key> : Poll_Time <value> : string representing the ping time to see if device is present (in milliseconds). |

Table 5 - Command Feedback Definitions

## Device Notes

PLEASE NOTE THAT THIS DUET PROJECT IS MEANT FOR RAPID PROJECT

MAKER (RPM) USE.  IF ATTEMPTING TO USE THIS PROJECT IN A

NON-RPM INSTALLATION YOU MUST USE THE APPROPRIATE AUDIO

ARCHITECT CONFIGURATION FILE (BLU_50_RPM_Configuration_v1.0.0.audioarchitect) OR IT WILL NOT WORK PROPERLY!

## Programming Notes

PLEASE NOTE: In order to establish a communication connection between the module and the device, the module must know what IP address to connect to. Use the PROPERTY- command to set the module’s IP address and then use the REINIT command to force it to take affect. Here is an example of how these two commands are used:

Send_Command 41001:1:0, ‘PROPERTY-IP_Address,192.168.103.30’

Send_Command 41001:1:0, ‘REINIT’

Note that your virtual device (41001:1:0) and the IP address of your device (192.168.103.30) may differ from this example. Substitute the appropriate values where necessary.

## Protocol Notes

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.  This is the ‘PASSTHRU-‘ command, which allows protocol strings to be passed through the module.  The device-specific protocol must be known in order to use this feature.

As an example, suppose that a module for a projector has not implemented the 'white balance adjustment' feature.   The command that the projector protocol requires is 03H, 10H, 05H, 14H, followed by a checksum.   The documentation for the ‘PASSTHRU-‘ command specifies that the module will automatically generate the checksum.   In this case, the following string should be sent from the UI code to implement 'white balance adjustment'.

send_command vdvBSSBLU50,”'PASSTHRU-‘,$88,$6b,$5c,$03,$00,$01,$1a,$00,$61,$00,$00,$00,$00"

The reason to use ‘PASSTHRU-‘ instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the ‘PASSTHRU-‘ command like checksum calculation.)