Duet Module Interface Specification

for the

Roku SettopBox

(IP Controlled)

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	4

Port Mapping	5

Channels	6

Command Control	7

Command Feedback	9

Device Notes	10

Programming Notes	10

Adding Functions to Modules	11

Commands to the device	11

Responses from the device	11

Revision History

| ###### Date | Initials | Version | Comments |
| --- | --- | --- | --- |
| 03-12-2017 | JL | v1.0.0 | Initial Release. |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a Roku Settop Box. The device supports IP based HTTP. The interface was tested using version 7.5.0 Build 4099 of the firmware.

This module was written using Café Duet firmware version 3.50.439, NetLinx Studio version 4.1 build 4.1.1203, Café Duet application platform and runtime version 1.15.230, and Café Duet application plug-in version 1.8.85.

For IP control, the Comm. module must be informed of the IP address of the device it is to connect to. This must be done manually via a telnet session, or can be automated from the UI module. The IP address must be set using the ‘PROPERTY-‘ command.

## Overview

The COMM module translates between the standard interface described below and the Roku network protocol. It parses the buffer for responses from the Roku, sends strings to control the Roku, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the command responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the Duet module.

Roku

Duet

COMM Module

NetLinx

UI Module

Virtual

Device

SNAPI

Some functionality in the device interface may not be implemented in the API interface.  In cases where device functions are desired but not API-supported, the PASSTHRU command may be used to send any and all device-protocol commands to the device. See the PASSTHRU command and the Adding Functions to Modules section for more information.

A sample UI module and a touch panel file are provided in the module package.  These are not intended to cover every possible application, but can be expanded as needed by a dealer to meet the requirements of a particular installation.

## Implementation

To interface to the AMX Roku module, the programmer must perform the following steps:

- Define the device ID for the Roku that will be controlled.

- Define the virtual device ID that the Roku COMM module will use to communicate with the main program and User Interface. Duet virtual devices use device numbers 41000 - 42000.

- If a touch panel interface is desired, a touch panel file Roku.TP4 and module ( Roku Main.axs) have been created for testing.

- The Duet Roku_Comm_dr1_0_0.jar module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the Roku to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

vdvRoku = 41001:1:0  // The COMM module should use this as its duet device

dvRoku = 0:3:0 // This device should be used as the physical device by the COMM module

dvRokuTp = 10001:1:0

(***********************************************************)

(*               CONSTANT DEFINITIONS GO BELOW             *)

(***********************************************************)

DEFINE_CONSTANT

(***********************************************************)

(*               VARIABLE DEFINITIONS GO BELOW             *)

(***********************************************************)

DEFINE_VARIABLE

DEV vdvDev[] = {vdvRoku}

// ----------------------------------------------------------

// CURRENT DEVICE NUMBER ON TP NAVIGATION BAR

INTEGER nRoku = 1

// ----------------------------------------------------------

// DEFINE THE PAGES THAT YOUR COMPONENTS ARE USING IN THE

// SUB NAVIGATION BAR HERE

INTEGER nTunerStationPages[] = {2}

INTEGER nSourceSelectPages[] = {3}

INTEGER nMenuPages[] = {4,5,6}

INTEGER nPowerPages[] = {8}

INTEGER nModulePages[] = {8}

INTEGER nDVRTransportPages[] = {9,10}

(***********************************************************)

(*                STARTUP CODE GOES BELOW                  *)

(***********************************************************)

DEFINE_START

// ----------------------------------------------------------

// DEVICE MODULE GROUPS SHOULD ALL HAVE THE SAME DEVICE NUMBER

DEFINE_MODULE 'Roku MenuComponent' menu(vdvDev, dvRokuTp, nRoku, nMenuPages)

DEFINE_MODULE 'Roku ModuleComponent' module(vdvDev, dvRokuTp, nRoku, nModulePages)

DEFINE_MODULE 'Roku PowerComponent' power(vdvDev, dvRokuTp, nRoku, nPowerPages)

DEFINE_MODULE 'Roku TunerStationComponent' tunerstation(vdvDev, dvRokuTp, nRoku, nTunerStationPages)

DEFINE_MODULE 'Roku DiscTransportComponent' disctransport(vdvDev, dvRokuTp, nRoku, nDVRTransportPages)

DEFINE_MODULE 'Roku SourceSelectComponent' sourceselect(vdvDev, dvRokuTp, nRoku, nSourceSelectPages)

// Define your communications module here like so:

DEFINE_MODULE 'Roku_Comm_dr1_0_0' comm(vdvRoku, dvRoku)

Upon initialization the AMX Comm module will communicate with the Roku and information will be exchanged.

## Port Mapping

| Virtual Device | Channels | Levels | Control | Feedback |
| --- | --- | --- | --- | --- |
| 41001:1:0 | All Channels | All Levels | All Control Commands | All Feedback Commands |

Table 1 - Port Mapping

## Channels

The UI module controls the Roku via channel events (NetLinx commands pulse, on, and off) sent to the COMM module. The channels supported by the COMM module are listed below. These channels are associated with the virtual device(s) and are independent of the channels associated with the touch panel device.

| Channel | Description |
| --- | --- |
| 1 | PULSE: Play/Pause |
| 6 | PULSE: Scan Forward |
| 7 | PULSE: Scan Reverse |
| 10 | PULSE: 0 Digit Button |
| 11 | PULSE: 1 Digit Button |
| 12 | PULSE: 2 Digit Button |
| 13 | PULSE: 3 Digit Button |
| 14 | PULSE: 4 Digit Button |
| 15 | PULSE: 5 Digit Button |
| 16 | PULSE: 6 Digit Button |
| 17 | PULSE: 7 Digit Button |
| 18 | PULSE: 8 Digit Button |
| 19 | PULSE: 9 Digit Button |
| 21 | PULSE: Enter Button |
| 44 | PULSE: Menu Button |
| 45 | PULSE: Move Menu Cursor Up |
| 46 | PULSE: Move Menu Cursor Down |
| 47 | PULSE: Move Menu Cursor Left |
| 48 | PULSE: Move Menu Cursor Right |
| 49 | PULSE: Select Menu Item |
| 80 | PULSE: Clear (Backspace on device) |
| 81 | PULSE: Back Button |
| 87 | PULSE: Left Parenthesis Button |
| 88 | PULSE: Right Parenthesis Button |
| 89 | PULSE: Underscore Button |
| 90 | PULSE: Dash Button |
| 91 | PULSE: Asterisk Button |
| 92 | PULSE: Dot Button |
| 101 | PULSE: Info Button |
| 196 | PULSE: Cycle Inputs (Apps) |
| 218 | PULSE: Instant Replay |
| 251 | ON: Device is Online – used for feedback only OFF: Device is not Online |
| 252 | ON: Data is Initialized – use for feedback only OFF: Data is not Initialized |
| 255 | ON: Power on – used for feedback OFF: Power off – used for feedback |

Table 2 - Virtual Device Channel Events

## Command Control

The UI module controls the Roku via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

Note: An ‘*’ indicates an extension to the standard API.

| Command | Description |
| --- | --- |
| ALPHA-<value> | Menu button A thru Z. Sends the Lit_ command to the device ALPHA-R |
| *?CHANNEL_NAME | Request the currently tuned Channel call sign. ?CHANNEL_NAME |
| ?DEBUG | Request the state of the debug feature. ?DEBUG |
| DEBUG-<value> | Set the state of debugging messages in the UI module and the Comm. module. Note: See Programming Notes section. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning & info messages on 4 = set all messages on DEBUG-1 |
| ?FWVERSION | Get the device’s firmware version. ?FWVERSION |
| ?INPUTPROPERTIES | Query for properties of all inputs. SEE NOTES! ?INPUTPROPERTIES |
| ?INPUTSELECT | Query for currently selected input. SEE NOTES! ?INPUTSELECT |
| INPUTSELECT-<index> | Set input selection. <index> : The input to select. Inputs are dynamically loaded based on available apps from the device – see notes for more details. INPUTSELECT-1 |
| PASSBACK-<state> | Enable or disable response reporting from the device. When enabled device responses will be sent as strings to the virtual device. Note: By default, this is set to off at startup. <state> : 0 = Off (default) 1 = On PASSBACK-0 |
| PASSTHRU-<string> | Allows user the capability of sending commands directly to whatever unit is attached with minimal processing by the Duet module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features that may not be directly supported by the module. For more information, see the “Adding Functions to Modules” section below. <string> : string to send to unit ‘passthru-search/browse?title=the%20neverending%20story’ |
| ?PROPERTY-<key> | Get the value of a property <key>. If the value is not initialized, an empty string is returned. <key> : IP_Address ?PROPERTY- IP_Address |
| PROPERTY-<key>,<value> | Set the value of property <key> to <value>. This must be followed by the REINIT command to take effect. These values are not initialized by default. <key> : IP_Address <value> : any valid IP address (default: 9.9.9.9) PROPERTY- IP_Address,9.9.9.9 |
| REINIT | Re-initializes the communication link and data. Note: This command deletes any messages waiting to go out to the device. REINIT |
| *SEARCH-<value> | Send a keyword search command to the device. Any spaces in the search will automatically be converted to %20 per device requirements. SEARCH-LILO AND STITCH |
| *?TITLE | Request the currently tuned show’s title. ?TITLE |
| ?XCH | Get the current tuner station. ?XCH |
| XCH-<station> | Set the current tuner station. <station> :Tuner station XCH-306.1 XCH-22 |

Table 3 – Send Command Definitions

## Command Feedback

The COMM module provides feedback to the User Interface module for Roku changes via command events. The commands supported are listed below.

PLEASE NOTE: Feedback is only provided when there is a state change. If no state change resulted from the command sent in, then no feedback will be returned.

| Command | Description |
| --- | --- |
| *CHANNEL_NAME | Returns the currently tuned Channel call sign. CHANNEL_NAME-getTV |
| DEBUG-<value> | Returns the state of debugging messages in the UI module and the Comm. module. <value> : 1 = set only error messages on 2 = set error and warning messages on 3 = set error, warning and info messages on 4 = set all messages on DEBUG-1 |
| FWVERSION-<version> | Returns the device’s firmware version. <version> : x.y = firmware version number FWVERSION=7.5.0 Build 4099 |
| INPUTCOUNT-<number> | Reports the number of currently available inputs. <number> : dynamic based on the available apps – see notes. INPUTCOUNT-x |
| INPUTPROPERTIES- <index>, <input group>, <signal type>, <display label>, <display name> | <index> : 1..x = Input index <input group> : 1..x = Input group index <signal type> : none or invalid = Type of signal <display label> : string = Label name of input <display name> : string = Display-friendly name of Input MAIN AS THE MENU SCREEN DOES NOT REPORT AS A KNOWN APP – THIS IS NOT A SELECTABLE INPUT – FEEDBACK ONLY. INPUTPROPERTIES-"1,1,INVALID,UNKNOWN APP,UNKNOWN APP" SAMPLE OF OTHERS – THESE ARE DYNAMICALLY LOADED BASED ON AVAILABLE DEVICE APPS. LISTING WILL VARY PER DEVICE! SEE NOTES FOR MORE INFORMATION! INPUTPROPERTIES-"3,3,NONE,Roku Home News,Roku Home News" INPUTPROPERTIES-"4,4,NONE,Netflix,Netflix" INPUTPROPERTIES-"5,5,NONE,Hulu,Hulu" INPUTPROPERTIES-"6,6,NONE,Amazon Video,Amazon Video" INPUTPROPERTIES-"7,7,NONE,Sling TV ,Sling TV " |
| INPUTPROPERTY- <index>, <input group>, <signal type>, <display label>, <display name> | <index> : 1..x = Input index <input group> : 1..x = Input group index <signal type> : none or invalid = Type of signal <display label>: string = Label name of input <display name> : string = Display-friendly name of Input Port 1 INPUTPROPERTY-"1,1,INVALID,UNKNOWN APP,UNKNOWN APP" |
| INPUTSELECT-<index> | Reports the currently selected input. <index> : 1..x INPUTSELECT-3 |
| PROPERTY-<key>,<value> | Feedback on the value of property <key>. Note: An empty string is returned if the property has no value. <key> : IP_Address <value> : any valid IP address (default: 9.9.9.9) PROPERTY-IP_Address,9.9.9.9 |
| *RATING-<rating> | Returns the currently tuned show’s rating. <rating> : String representing Rating RATING-TV-14-D-V |
| *TITLE-<title> | Request the currently tuned show’s title. TITLE-Airwolf |
| XCH-<station> | Returns the current tuner station. <station> :Tuner Station XCH-14.3 XCH-2 |

Table 4 - Command Feedback Definitions

## Device Notes

- The device does not support unsolicited feedback.

- Module validation was performed with a Roku 2 4210X Firmware: 7.5.0 Build 4099.  Module TV support was not verified as device does not support TV.

- As not all Roku devices support TV, there is no automated polling of TV information.  To query the current station on the device use the module query channel “?XCH” command.  This will query the device and then update the module.

- Any device command that is an HTTP POST provides no device feedback.  Device commands that provide feedback are HTTP GET commands.  As the search command is an HTTP POST command, the only feedback to the user is what is visible on the monitor screen.

- Passthru commands are assumed to be HTTP POST commands.  To send an HTTP GET command format the command as follows: PASSTHRU-(GET)<value>.  Example: PASSTHRU-(GET)query/active-app

- To perform searches the Roku must have search enabled set to true.  The module reads this setting value in the device-info and will print a warning should the value not be true prior to sending the search command to the device.  There is no known command to set this parameter through the module.

- There is no power on command documented in the device API.  As such power function is not supported.  Power state will be read from the device response and returned to the user.

## Programming Notes

- There is no query or feedback to get the current DVR transport state. The “PLAY” button also appears to be “PAUSE” and the status of the “PLAY”, “REV” and “FWD” are unknown at all times.

- The “sources” are unknown until the module is able to query for the available installed apps on the device.  Once feedback from the device is provided the module will dynamically populate the source inputs.  Source 1 is set as the “UNKNOWN APP” to provide the user feedback that the device is in an unknown app (example the Roku menu).

- Each time the module receives the query/apps response, the app inputs will be removed and new listing added causing a module refresh to occur.  This is to accommodate any changes that may be seen in the device app listing and is expected.  Please note that during testing the response would sometimes multiply during initialization – it appears as though the device queues the queries and responds to all at once causing the module to refresh multiple times

- The sample touch panel project is limited to 20 dynamic app results.  If more are required please manually add more slots to the sample UI code and touch panel.

## Adding Functions to Modules

### Commands to the device

This module supplies a mechanism to allow additional device features to be added to software using the module.  This is the ‘PASSTHRU-‘ command, which allows protocol strings to be passed through the module.  The device-specific protocol must be known in order to use this feature.

As an example, to send a search command for title ‘the neverending story’ the command that the protocol requires is http://IP:port/search/browse?title=the%20neverending%20story.  The module automatically creates the beginning of the string up to the port. In this case, the following string should be sent from the UI code:

send_command vdvDevice, 'PASSTHRU-search/browse?title=the%20neverending%20story’

All passthru commands are assumed to be HTTP POST commands unless the module passthru command is informed otherwise.  To send an HTTP GET passthru command format as follows: ‘PASSTHRU-(GET)<value>’.  Example:

send_command vdvDevice, 'PASSTHRU-(GET)query/active-app’

The reason to use ‘PASSTHRU-‘ instead of sending a protocol string directly to the device port is that the device may require command queuing, calculation of checksums, or other internal processing, which would not be done if the string was sent directly. Because of this, it is best to filter all communication TO the device through the module. (The module documentation will indicate any processing that will be automatically done to the ‘PASSTHRU-‘ command like checksum calculation.)

### Responses from the device

The module will automatically interpret replies from the device and pass these on to the application code according to the documented API.  Some device replies may not be passed on to the application code.

To see all replies from the device unfiltered by the module, enable PASSBACK and use a DATA_EVENT with a string handler in the UI code.  Again, the device-specific protocol must be known in order to interpret these responses.  Even when PASSBACK is enabled, the module will still interpret device responses according to the standard API.